Features：

·SOI H65 CMOS Image Sensor

· Full HD-720P

· 2D-NR

Parameter

| Specifications | Model | TL-DMS-S2 |
| --- | --- | --- |
|  | Name | DMS Camera |
|  | Sensor | H65 |
|  | Image Sensor | 130W CMOS Sensor |
|  | Image color | Black -White |
|  | Focal Length | 3.6mm |
|  | IR | NA |
|  | View Angle | D=85° H=60° V=53° |
|  | WDR | Yes |
|  | Signal Type | PAL |
|  | FPS | 25fps |
| Properties | Temperature | -25℃~75℃ |
|  | Humidity | ≤ 90 % |
|  | Voltage | 12V DC |
|  | Rated power | 1.25W |
|  | Cable Length | Aviation connector, 2.5m |

Pin Feet Definition

| PIN | Name | Pin Type | Function |
| --- | --- | --- | --- |
| 1 | VCC | Power | 12V |
| 2 | GND | Power | Cathode |
| 3 | NA | NA | NA |
| 4 | VIDEO | AHD | Video |